Lab 7

Part 1.

“include "ram32x4.v”

module lab7p1 (input [9:0] SW, input [1:0] KEY, output [6:0] HEX0, output [6:0] HEX2, output [6:0] HEX4, output [6:0] HEX5);
wire [3:0] data;
assign data[3:0] = SW[3:0];
wire [4:0] address;
assign address[4:0] = SW[8:4];
wire wren;
wire clock;
assign wren = SW[9];
assign clock = KEY[0];

wire out_value;

ram32x4 ram_ist(
.address(address),
.clock(clock),
.data(data),
.wren(wren),
.q(out_value)
);

// === Display the output

// the data output of the memory
hex_decoder H0( 
.hex_digit(out_value),
.segments(HEX0)
);

// the data input of the memory 
hex_decoder H2(
.hex_digit(data),
.segments(HEX2)
);

// Display the address 
hex_decoder H4( 
.hex_digit(address[3:0]),
.segments(HEX0)
);

// Display the remaining bit of the address. 
hex_decoder H5(
.hex_digit(address[4]),
.segments(HEX1)
);

endmodule

module hex_decoder(input [3:0] hex_digit, output reg [6:0]segments);
always @(*)
case (hex_digit)
4'h0: segments = 7'b100_0000;
4'h1: segments = 7'b111_1001;
4'h2: segments = 7'b010_0100;
4'h3: segments = 7'b011_0000;
4'h4: segments = 7'b001_1001;
4'h5: segments = 7'b001_0010;
4'h6: segments = 7'b000_0010;
4'h7: segments = 7'b111_1000;
4'h8: segments = 7'b000_0000;
4'h9: segments = 7'b001_1000;
4'hA: segments = 7'b000_1000;
4'hB: segments = 7'b000_0011;
4'hC: segments = 7'b100_0110;
4'hD: segments = 7'b010_0001;
4'hE: segments = 7'b000_0110;
4'hF: segments = 7'b000_1110;
default: segments = 7'h7f;
endcase
endmodule

Part 2.

- Datapath

module datapath(
input ld_x, ld_y, ld_colour,
input start_draw,
input clock,
input resetn,
input [9:0] data_in,
input [3:0] counter,
output reg [2:0] colour,
output reg [7:0] x,
output reg [6:0] y
);
reg [7:0]x_original;
reg [6:0]y_original;
always @ (posedge clock) begin
if (!resetn) begin
x <= 8'b00000000; 
y <= 7'b0000000; 
colour <= 3'b000;
x_original <= 8'b00000000;
y_original <= 7'b0000000;
end
else begin
if (ld_x)begin
x_original <= {1'b0, data_in[6:0]};
x <= {1'b0, data_in[6:0]};
end
if (ld_y && ld_colour) begin
y <= data_in[6:0]; 
y_original <= data_in[6:0]; 
colour <= data_in[9:7];
end
if (start_draw) begin
x <= x_original + counter[1:0];
y <= y_original + counter[3:2];
end
end
end
endmodule

2. FSM

module FSMwithdatapath
(
CLOCK_50,
KEY,
SW
);

input CLOCK_50; // 50 MHz
input [9:0] SW;
input [3:0] KEY;

// Declare your inputs and outputs here
// Do not change the following outputs
wire resetn;
assign resetn = KEY[0];
wire load_x;
assign load_x = ~KEY[3];
wire load_y;
assign load_y = ~KEY[1];
wire ld_x,ld_y,ld_colour,start_draw;
wire [3:0] counter;
// Create the colour, x, y and writeEn wires that are inputs to the controller.
wire [2:0] colour;
wire [7:0] x;
wire [6:0] y;
wire writeEn;

// Create an Instance of a VGA controller - there can be only one!
// Define the number of colours as well as the initial background
// image file (.MIF) for the controller.
// Put your code here. Your code should produce signals x,y,colour and writeEn/plot
// for the VGA controller, in addition to any other functionality your design may require.

// Instansiate datapath
datapath d0(
.ld_x(ld_x),
.ld_y(ld_y),
.ld_colour(ld_colour),
.start_draw(start_draw),
.resetn(resetn),
.clock(CLOCK_50),
.data_in(SW[9:0]),
.counter(counter),
.colour(colour),
.x(x),
.y(y)
);

// Instansiate FSM control
control c0(
.resetn(resetn),
.clock(CLOCK_50),
.load_x(load_x),
.load_y(load_y),
.ld_x(ld_x),
.ld_y(ld_y),
.ld_colour(ld_colour),
.start_draw(start_draw),
.plot(writeEn),
.counter(counter)
);

endmodule

module control(
input resetn,
input clock,
input load_x,
input load_y,
output reg ld_x, ld_y, ld_colour,
output reg start_draw,
output reg plot,
output reg [3:0] counter = 4'b0001
);
reg [3:0] current_state, next_state;
reg done;

localparam LOAD_X_WAIT = 4'd0,
LOAD_X = 4'd1,
LOAD_Y_WAIT = 4'd2,
LOAD_Y_COLOUR = 4'd3,
DRAW = 4'd4;
always @(*)
begin: state_table 
case (current_state)
LOAD_X_WAIT: next_state = load_x ? LOAD_X : LOAD_X_WAIT; // waiting for the load_x to become high
LOAD_X: next_state = load_x ? LOAD_X : LOAD_Y_WAIT; // load the x value into datapath until load_x become low
LOAD_Y_WAIT: next_state = load_y ? LOAD_Y_COLOUR : LOAD_Y_WAIT; // waiting for the load_y to become high
LOAD_Y_COLOUR: next_state = load_y ? LOAD_Y_COLOUR : DRAW; // load y and colour into datapath, start drawing when load_y become low
DRAW: next_state = done ? LOAD_X_WAIT : DRAW; // we had told the datapath it can start drawing and we are now waiting for the new X input.
default: next_state = LOAD_X_WAIT;
endcase
end 

always @(*)
begin: enable_signals
ld_x = 1'b0;
ld_y = 1'b0;
ld_colour = 1'b0;
start_draw = 1'b0;
plot = 1'b0;

case (current_state)
LOAD_X: begin
ld_x = 1'b1;
end
LOAD_Y_COLOUR: begin
ld_y = 1'b1;
ld_colour = 1'b1;
end
DRAW: begin
start_draw = 1'b1;
plot = 1'b1;
end
endcase
end 

always@(posedge clock)
begin
if(!resetn) begin
counter <= 4'b0001;
done <= 1'b0;
end
else 
if (current_state == DRAW) begin
if (counter == 4'b1111) 
done <= 1'b1;
counter <= counter + 1'b1;
end 
else
done <= 1'b0;
end 

always@(posedge clock)
begin: state_FFs
if(!resetn)
current_state <= LOAD_X_WAIT;
else
current_state <= next_state;
end // state_FFS
endmodule

module datapath(
input ld_x, ld_y, ld_colour,
input start_draw,
input clock,
input resetn,
input [9:0] data_in,
input [3:0] counter,
output reg [2:0] colour,
output reg [7:0] x,
output reg [6:0] y
);
reg [7:0]x_original;
reg [6:0]y_original;
always @ (posedge clock) begin
if (!resetn) begin
x <= 8'b00000000; 
y <= 7'b0000000; 
colour <= 3'b000;
x_original <= 8'b00000000;
y_original <= 7'b0000000;
end
else begin
if (ld_x)begin
x_original <= {1'b0, data_in[6:0]};
x <= {1'b0, data_in[6:0]};
end
if (ld_y && ld_colour) begin
y <= data_in[6:0]; 
y_original <= data_in[6:0]; 
colour <= data_in[9:7];
end
if (start_draw) begin
x <= x_original + counter[1:0];
y <= y_original + counter[3:2];
end
end
end
endmodule

- FSM

module FSM(resetn, clock, load_x, load_y, ld_x, ld_y, ld_colour, start_draw, plot, counter);
input resetn;
input clock;
input load_x;
input load_y;
output reg ld_x, ld_y, ld_colour;
output reg start_draw;
output reg plot;
output reg [3:0] counter = 4'b0001;

reg [3:0] current_state, next_state;
reg done;

localparam LOAD_X_WAIT = 4'd0,
LOAD_X = 4'd1,
LOAD_Y_WAIT = 4'd2,
LOAD_Y_COLOUR = 4'd3,
DRAW = 4'd4;
always @(*)
begin: state_table 
case (current_state)
LOAD_X_WAIT: next_state = load_x ? LOAD_X : LOAD_X_WAIT; // waiting for the load_x to become high
LOAD_X: next_state = load_x ? LOAD_X : LOAD_Y_WAIT; // load the x value into datapath until load_x become low
LOAD_Y_WAIT: next_state = load_y ? LOAD_Y_COLOUR : LOAD_Y_WAIT; // waiting for the load_y to become high
LOAD_Y_COLOUR: next_state = load_y ? LOAD_Y_COLOUR : DRAW; // load y and colour into datapath, start drawing when load_y become low
DRAW: next_state = done ? LOAD_X_WAIT : DRAW; // we had told the datapath it can start drawing and we are now waiting for the new X input.
default: next_state = LOAD_X_WAIT;
endcase
end 

always @(*)
begin: enable_signals
ld_x = 1'b0;
ld_y = 1'b0;
ld_colour = 1'b0;
start_draw = 1'b0;
plot = 1'b0;

case (current_state)
LOAD_X: begin
ld_x = 1'b1;
end
LOAD_Y_COLOUR: begin
ld_y = 1'b1;
ld_colour = 1'b1;
end
DRAW: begin
start_draw = 1'b1;
plot = 1'b1;
end
endcase
end 

always@(posedge clock)
begin
if(!resetn) begin
counter <= 4'b0001;
done <= 1'b0;
end
else 
if (current_state == DRAW) begin
if (counter == 4'b1111) 
done <= 1'b1;
counter <= counter + 1'b1;
end 
else
done <= 1'b0;
end 

always@(posedge clock)
begin: state_FFs
if(!resetn)
current_state <= LOAD_X_WAIT;
else
current_state <= next_state;
end // state_FFS
endmodule

3.

module part2
(
CLOCK_50, // On Board 50 MHz
// Your inputs and outputs here
KEY,
SW,
// The ports below are for the VGA output. Do not change.
VGA_CLK, // VGA Clock
VGA_HS, // VGA H_SYNC
VGA_VS, // VGA V_SYNC
VGA_BLANK_N, // VGA BLANK
VGA_SYNC_N, // VGA SYNC
VGA_R, // VGA Red[9:0]
VGA_G, // VGA Green[9:0]
VGA_B // VGA Blue[9:0]
);

input CLOCK_50; // 50 MHz
input [9:0] SW;
input [3:0] KEY;

// Declare your inputs and outputs here
// Do not change the following outputs
output VGA_CLK; // VGA Clock
output VGA_HS; // VGA H_SYNC
output VGA_VS; // VGA V_SYNC
output VGA_BLANK_N; // VGA BLANK
output VGA_SYNC_N; // VGA SYNC
output [9:0] VGA_R; // VGA Red[9:0]
output [9:0] VGA_G; // VGA Green[9:0]
output [9:0] VGA_B; // VGA Blue[9:0]

wire resetn;
assign resetn = KEY[0];
wire load_x;
assign load_x = ~KEY[3];
wire load_y;
assign load_y = ~KEY[1];
wire ld_x,ld_y,ld_colour,start_draw;
wire [3:0] counter;
// Create the colour, x, y and writeEn wires that are inputs to the controller.
wire [2:0] colour;
wire [7:0] x;
wire [6:0] y;
wire writeEn;

// Create an Instance of a VGA controller - there can be only one!
// Define the number of colours as well as the initial background
// image file (.MIF) for the controller.
vga_adapter VGA(
.resetn(resetn),
.clock(CLOCK_50),
.colour(colour),
.x(x),
.y(y),
.plot(writeEn),
/* Signals for the DAC to drive the monitor. */
.VGA_R(VGA_R),
.VGA_G(VGA_G),
.VGA_B(VGA_B),
.VGA_HS(VGA_HS),
.VGA_VS(VGA_VS),
.VGA_BLANK(VGA_BLANK_N),
.VGA_SYNC(VGA_SYNC_N),
.VGA_CLK(VGA_CLK));
defparam VGA.RESOLUTION = "160x120";
defparam VGA.MONOCHROME = "FALSE";
defparam VGA.BITS_PER_COLOUR_CHANNEL = 1;
defparam VGA.BACKGROUND_IMAGE = "black.mif";

// Put your code here. Your code should produce signals x,y,colour and writeEn/plot
// for the VGA controller, in addition to any other functionality your design may require.

// Instansiate datapath
datapath d0(
.ld_x(ld_x),
.ld_y(ld_y),
.ld_colour(ld_colour),
.start_draw(start_draw),
.resetn(resetn),
.clock(CLOCK_50),
.data_in(SW[9:0]),
.counter(counter),
.colour(colour),
.x(x),
.y(y)
);

// Instansiate FSM control
control c0(
.resetn(resetn),
.clock(CLOCK_50),
.load_x(load_x),
.load_y(load_y),
.ld_x(ld_x),
.ld_y(ld_y),
.ld_colour(ld_colour),
.start_draw(start_draw),
.plot(writeEn),
.counter(counter)
);

endmodule

module control(
input resetn,
input clock,
input load_x,
input load_y,
output reg ld_x, ld_y, ld_colour,
output reg start_draw,
output reg plot,
output reg [3:0] counter = 4'b0001
);
reg [3:0] current_state, next_state;
reg done;

localparam LOAD_X_WAIT = 4'd0,
LOAD_X = 4'd1,
LOAD_Y_WAIT = 4'd2,
LOAD_Y_COLOUR = 4'd3,
DRAW = 4'd4;
always @(*)
begin: state_table 
case (current_state)
LOAD_X_WAIT: next_state = load_x ? LOAD_X : LOAD_X_WAIT; // waiting for the load_x to become high
LOAD_X: next_state = load_x ? LOAD_X : LOAD_Y_WAIT; // load the x value into datapath until load_x become low
LOAD_Y_WAIT: next_state = load_y ? LOAD_Y_COLOUR : LOAD_Y_WAIT; // waiting for the load_y to become high
LOAD_Y_COLOUR: next_state = load_y ? LOAD_Y_COLOUR : DRAW; // load y and colour into datapath, start drawing when load_y become low
DRAW: next_state = done ? LOAD_X_WAIT : DRAW; // we had told the datapath it can start drawing and we are now waiting for the new X input.
default: next_state = LOAD_X_WAIT;
endcase
end 

always @(*)
begin: enable_signals
ld_x = 1'b0;
ld_y = 1'b0;
ld_colour = 1'b0;
start_draw = 1'b0;
plot = 1'b0;

case (current_state)
LOAD_X: begin
ld_x = 1'b1;
end
LOAD_Y_COLOUR: begin
ld_y = 1'b1;
ld_colour = 1'b1;
end
DRAW: begin
start_draw = 1'b1;
plot = 1'b1;
end
endcase
end 

always@(posedge clock)
begin
if(!resetn) begin
counter <= 4'b0001;
done <= 1'b0;
end
else 
if (current_state == DRAW) begin
if (counter == 4'b1111) 
done <= 1'b1;
counter <= counter + 1'b1;
end 
else
done <= 1'b0;
end 

always@(posedge clock)
begin: state_FFs
if(!resetn)
current_state <= LOAD_X_WAIT;
else
current_state <= next_state;
end // state_FFS
endmodule

module datapath(
input ld_x, ld_y, ld_colour,
input start_draw,
input clock,
input resetn,
input [9:0] data_in,
input [3:0] counter,
output reg [2:0] colour,
output reg [7:0] x,
output reg [6:0] y
);
reg [7:0]x_original;
reg [6:0]y_original;
always @ (posedge clock) begin
if (!resetn) begin
x <= 8'b00000000; 
y <= 7'b0000000; 
colour <= 3'b000;
x_original <= 8'b00000000;
y_original <= 7'b0000000;
end
else begin
if (ld_x)begin
x_original <= {1'b0, data_in[6:0]};
x <= {1'b0, data_in[6:0]};
end
if (ld_y && ld_colour) begin
y <= data_in[6:0]; 
y_original <= data_in[6:0]; 
colour <= data_in[9:7];
end
if (start_draw) begin
x <= x_original + counter[1:0];
y <= y_original + counter[3:2];
end
end
end
endmodule